ECA56 – ADVANCED WIRELESS COMMUNICATION SYSTEMS

EXPERIMENT 4

Measurement of Wireless Coverage and Channel Interference

Prepared by: G. Chennakesavareddy

AIM

To measure Wi-Fi RSSI and observe channel activity, then compare signal strength at different distances and identify a suitable Wi-Fi channel.

APPARATUS / SOFTWARE REQUIRED

ESP32 development board, USB cable, computer with Arduino IDE, Wi-Fi access point/router, and a phone or computer to view the router's channel settings. This experiment requires hardware for real RSSI measurements.

MATLAB / ARDUINO CODE

/*
 EXPERIMENT 4: ESP32 Wi-Fi RSSI and channel scan
 Board: ESP32
 Select the correct ESP32 board and COM port in Arduino IDE.
*/
#include <WiFi.h>

const char* ssid = "YOUR_WIFI_NAME";
const char* password = "YOUR_WIFI_PASSWORD";

void setup() {
  Serial.begin(115200);
  delay(1000);
  WiFi.mode(WIFI_STA);
  WiFi.begin(ssid, password);

  Serial.println("Connecting to Wi-Fi...");
  int attempts = 0;
  while (WiFi.status() != WL_CONNECTED && attempts < 30) {
    delay(500);
    Serial.print(".");
    attempts++;
  }

  if (WiFi.status() == WL_CONNECTED) {
    Serial.println("\nConnected!");
    Serial.print("IP address: ");
    Serial.println(WiFi.localIP());
  } else {
    Serial.println("\nConnection failed. Check credentials.");
  }
}

void loop() {
  if (WiFi.status() == WL_CONNECTED) {
    Serial.print("RSSI (dBm): ");
    Serial.println(WiFi.RSSI());
  }

  Serial.println("Scanning nearby Wi-Fi networks...");
  int n = WiFi.scanNetworks();
  for (int i = 0; i < n; i++) {
    Serial.print(i + 1);
    Serial.print(": ");
    Serial.print(WiFi.SSID(i));
    Serial.print(" | Channel: ");
    Serial.print(WiFi.channel(i));
    Serial.print(" | RSSI: ");
    Serial.print(WiFi.RSSI(i));
    Serial.println(" dBm");
  }
  WiFi.scanDelete();
  delay(5000);
}

OUTPUT SCREENSHOTS

No output screenshot was provided for this experiment yet. Capture the serial-monitor RSSI/channel scan output after running the hardware experiment and insert it here.

RESULT

Record RSSI at multiple distances and note the channels and signal levels reported by nearby networks. Replace the Wi-Fi placeholders with your own network credentials before uploading. Do not include your password in the submitted record.