ECA56 – ADVANCED WIRELESS COMMUNICATION SYSTEMS

EXPERIMENT 9

Comparison of BPSK, QPSK and 16-QAM

Prepared by: G. Chennakesavareddy

AIM

To compare the bit error rate (BER) of BPSK, QPSK and 16-QAM modulation schemes over an AWGN channel and display their constellation diagrams.

APPARATUS / SOFTWARE REQUIRED

Computer with MATLAB Online/Desktop. No physical hardware is required; this is a software simulation.

MATLAB / ARDUINO CODE

clc;
clear;
close all;
rng(7);

% EXPERIMENT 9: BPSK, QPSK and 16-QAM comparison
N = 100000;
SNR_dB = 0:2:20;
BER_BPSK = zeros(size(SNR_dB));
BER_QPSK = zeros(size(SNR_dB));
BER_16QAM = zeros(size(SNR_dB));

% BPSK data and symbols
bitsBPSK = randi([0 1],N,1);
txBPSK = 2*bitsBPSK - 1;

% QPSK data and symbols
bitsQPSK = randi([0 1],2*N,1);
b1 = bitsQPSK(1:2:end);
b2 = bitsQPSK(2:2:end);
txQPSK = ((1-2*b1) + 1j*(1-2*b2))/sqrt(2);

% 16-QAM Gray-like rectangular mapping, normalized average power
bits16 = randi([0 1],4*N,1);
B = reshape(bits16,4,[]).';
I = (1-2*B(:,1)).*(3-(1-2*B(:,2)));
Q = (1-2*B(:,3)).*(3-(1-2*B(:,4)));
tx16QAM = (I + 1j*Q)/sqrt(10);

for k = 1:length(SNR_dB)
    snrLin = 10^(SNR_dB(k)/10);

    % BPSK
    noise = sqrt(1/(2*snrLin))*randn(size(txBPSK));
    rx = txBPSK + noise;
    BER_BPSK(k) = mean((rx>0) ~= bitsBPSK);

    % QPSK
    noise = sqrt(1/(2*snrLin))*(randn(size(txQPSK))+1j*randn(size(txQPSK)));
    rx = txQPSK + noise;
    d1 = real(rx)<0;
    d2 = imag(rx)<0;
    detected = zeros(2*N,1);
    detected(1:2:end) = d1;
    detected(2:2:end) = d2;
    BER_QPSK(k) = mean(detected ~= bitsQPSK);

    % 16-QAM: nearest levels on each axis
    noise = sqrt(1/(2*snrLin))*(randn(size(tx16QAM))+1j*randn(size(tx16QAM)));
    rx = tx16QAM + noise;
    levels = [-3 -1 1 3]/sqrt(10);
    [~,idxI] = min(abs(real(rx)-levels),[],2);
    [~,idxQ] = min(abs(imag(rx)-levels),[],2);
    recI = levels(idxI);
    recQ = levels(idxQ);
    % Reconstruct bits using the same axis mapping
    recBits = zeros(N,4);
    recBits(:,1) = recI < 0;
    recBits(:,2) = abs(recI) < 2/sqrt(10);
    recBits(:,3) = recQ < 0;
    recBits(:,4) = abs(recQ) < 2/sqrt(10);
    BER_16QAM(k) = mean(recBits(:) ~= bits16);
end

BER_BPSK(BER_BPSK==0) = 0.5/N;
BER_QPSK(BER_QPSK==0) = 0.5/(2*N);
BER_16QAM(BER_16QAM==0) = 0.5/(4*N);

figure(1);
semilogy(SNR_dB,BER_BPSK,'o-','LineWidth',1.5);
hold on;
semilogy(SNR_dB,BER_QPSK,'s-','LineWidth',1.5);
semilogy(SNR_dB,BER_16QAM,'^-','LineWidth',1.5);
grid on;
xlabel('SNR (dB)');
ylabel('Bit Error Rate');
title('BER Comparison of BPSK, QPSK and 16-QAM');
legend('BPSK','QPSK','16-QAM','Location','best');

figure(2);
subplot(1,3,1);
plot(real(txBPSK(1:2000)),imag(txBPSK(1:2000)),'o');
grid on; axis equal; title('BPSK Constellation');
xlabel('In-Phase'); ylabel('Quadrature');

subplot(1,3,2);
plot(real(txQPSK(1:2000)),imag(txQPSK(1:2000)),'o');
grid on; axis equal; title('QPSK Constellation');
xlabel('In-Phase'); ylabel('Quadrature');

subplot(1,3,3);
plot(real(tx16QAM(1:2000)),imag(tx16QAM(1:2000)),'o');
grid on; axis equal; title('16-QAM Constellation');
xlabel('In-Phase'); ylabel('Quadrature');

OUTPUT SCREENSHOTS

RESULT

The BER curves and constellation diagrams compare the three modulation schemes. BPSK generally offers better BER robustness at a given SNR, while higher-order modulation carries more bits per symbol at the cost of greater noise sensitivity.